# 1. Current Setup (Using CANoe)

## Existing Workflow

Currently, CANoe is used for:

- Sending UDS diagnostic requests

- Receiving ECU responses

- Verifying CAN and LIN communication

- Performing basic validation tests

- Monitoring bus communication

## Typical Workflow

CANoe → USB Interface → CAN/LIN Bus → ECU

## Limitations in Current Setup

### 1.1 High Licensing Cost

- CANoe requires expensive licenses

- Multiple stations increase cost significantly

- Annual maintenance costs apply

### 1.2 Limited Scalability

- Each production station requires a license

- License management overhead exists

### 1.3 Heavy Software Dependency

- Requires trained personnel

- Not lightweight for automation workflows

### 1.4 Limited Automation Flexibility

- CAPL-based scripting requires expertise

- Integration with external systems is complex

# 2. Proposed Solution

Replace CANoe-based communication with a Python-based diagnostic and validation tool using USB CAN/LIN hardware.

## Proposed Hardware

Primary Interface:

- Kvaser Hybrid 2xCAN/LIN Interface

# 3. Proposed Software Architecture

The proposed solution uses open-source Python libraries along with vendor-provided drivers.

## Software Stack

- Python 3.x

- python-can

- udsoncan

- isotp

- Kvaser CANlib drivers

- Kvaser LINlib drivers

## System Architecture

Production PC │ ├── Python Diagnostic Tool │ │ │ ├── python-can │ ├── udsoncan │ ├── isotp │ ├── USB Interface (Kvaser Hybrid 2xCAN/LIN) │ │ │ ├── CAN Network │ └── LIN Network │ └── NFC Reader (Optional)

# 4. Functional Capability Comparison

| Feature | CANoe | Proposed Python Tool |
| --- | --- | --- |
| Send UDS Request | Supported | Supported |
| Receive Response | Supported | Supported |
| CAN Communication | Supported | Supported |
| LIN Communication | Supported | Supported |
| Logging | Supported | Supported |
| Automation | Limited | Highly Flexible |
| Licensing | Required | Not Required |
| Scalability | Limited | High |

# 5. Typical Use Cases Covered

The proposed system supports the following:

## 5.1 UDS Communication

Examples:

- Diagnostic Session Control (0x10)

- Read Data by Identifier (0x22)

- Security Access (0x27)

- ECU Reset (0x11)

## 5.2 ECU Validation

- Verify ECU communication status

- Validate CAN response

- Validate LIN response

- Confirm NFC functionality

## 5.3 Production Testing

- Send automated test commands

- Validate response data

- Generate pass/fail results

## 5.4 ECU Flashing (Future Use)

- Bootloader communication

- Firmware transfer

- Verification

# 6. Migration Plan

## Step 1 — Hardware Procurement

Purchase:

- Kvaser Hybrid 2xCAN/LIN

## Step 2 — Software Setup

Install:

- Python environment

- Required libraries

- Kvaser drivers

## Step 3 — Initial Validation

Test:

- Basic CAN communication

- Basic LIN communication

- UDS command exchange

## Step 4 — Test Case Development

Create scripts for:

- ECU validation

- Communication verification

## Step 5 — Parallel Testing

Run both:

- CANoe

- Python tool

Compare results.